| Device No.: | Version: | Revision State: | Author | Date |
| --- | --- | --- | --- | --- |
| 261503933xxxx | HW-V1 | 1 | JuSa | 19.04.2023 |

PCB must comply with IPC-A-600 Class 2.

PCBs must comply with Class V1 of EN60950 for flammability and temperature resistance.

PCB-Manufacture must correspond to the specifications of Würth Elektronik eiSos.

PCBs electrical tested.

PCBs must meet RoHS and Reach requirements and must be Halogen free.

| Dimensions: | 80,0x22,86mm |  | Number of Layers: | 4 |  |
| --- | --- | --- | --- | --- | --- |
| Tolerances: | +/- 0,2mm |  |  |  |  |
| After Separating (EMS) +/-0,4mm |  |  |  |  |  |
| Multiple PCB Panel: | Panel chosen by assembling service mm |  | PCBs per Panel |  |  |
|  |  |  |  | Step & repeat through Manufacturer |  |
| Nominal thickness: | 1,67 mm |  |  |  |  |

| Material Type: |  | FR4 (135°TG) |  |  | FR4 (150°TG) |  | FR4 (170°TG) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| unfilled |  |  |  |  |  |  |  |
| Separation |  | Milling |  |  | Scribing / V-Nut |  | jump |
| Surface |  | HAL lead-free |  |  | chem. Ni/Au |  | chem. Sn |

| Tin coated edges | Number of contacts |  |  | ## Header |  |
| --- | --- | --- | --- | --- | --- |
|  | 0 |  |  | Angular |  |
|  | Technique: cut vias |  |  | Galvanic gold |  |

| Print |  | white |  | Layer |  | 112 tprint |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | 26 bnames |

| Solder mask |  | top & bottom |  |
| --- | --- | --- | --- |
| Color of SM |  | rot |  |

| Drill-Scheme | Final Diameter mm | Tol. | Aspect ratio | Number / PCB |
| --- | --- | --- | --- | --- |
| Via smallest drill | 0,2 |  |  | 447 |
| Plated through hole | 0,2 |  |  | 4 |
| Plated through hole | 0,3 |  |  | 3 |
| Plated through hole | 0,55 |  |  | 2 |
| Plated through hole | 0,65 |  |  | 4 |
| Plated through hole | 0,85 |  |  | 2 |
| Plated through hole | 1 |  |  | 28 |
| Plated through hole | 2,5 |  |  | 2 |
| Non-plated through hole | 0,8 |  |  | 2 |
| Non-plated through hole | 2,54 |  |  | 2 |

Stack Up:

| Layer | Type | Material | Min. Width mm | min. Distance mm | min Restring mm | Thickness µm |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Top | Cu | 0,15 | 0,15 | 0,15 | 35 |
|  |  | FR4 |  |  |  | 410 |
| 2 | Route2 | Cu | 0,15 | 0,15 | 0,15 | 35 |
|  |  | FR4 |  |  |  | 710 |
| 15 | Route15 | Cu | 0,15 | 0,15 | 0,15 | 35 |
|  |  | FR4 |  |  |  | 410 |
| 16 | Bottom | Cu | 0,15 | 0,15 | 0,15 | 35 |

Change of Material should be avoided. If not avoidable it has to be announced in advance and new samples will be necessary.

Note:

- Würth Elektronik Gerber data show the required final dimensions and positions of all structures. No compensations for process related deviations have been made to Gerber data. The supplier shall deliver the PCB with structural dimensions as defined in Würth Elektronik Gerber data.

Exception: solder resist at vias adapted by manufacture to cut outs around the drill, so that the pads are coated as complete as possible without risking of solder resist running into the vias.

- Tin plated Edge pads:

  - No separation:

  - No chips:

  - No metal between the pads:

Verwendete EAGLE Version:	7.7.0

Lagenzuordnung:

| Funktion der Lage | Lagenname EAGLE | Lagennr. EAGLE | Abw. Lagenname | Abw. Lagennr. | Verw. |
| --- | --- | --- | --- | --- | --- |
| Toplayer | Top | 1 | - | - | Ja |
| Innenlage 1 | Route2 | 2 | - | - | Ja |
| Innenlage 2 | Route15 | 15 | - | - | Ja |
| Bottomlayer | Bottom | 16 | - | - | Ja |
| Lötaugen | Pads | 17 | - | - | Ja |
| Durchsteiger | Vias | 18 | - | - | Ja |
| Außenkontur | Dimension | 20 | - | - | Ja |
| Bestückungsdruck Top | tprint | 112 | - | - | Ja |
| Bestückungsdruck Bottom | bnames | 26 | - | - | Ja |
| Lötstopp Top | tStop | 29 | - | - | Ja |
| Lötstopp Bottom | bStop | 30 | - | - | Ja |
| Paste Top | tCream | 31 | - | - | Ja |
| Paste Bottom | bCream | 32 | - | - | Ja |
| Klebung Top | tGlue | 35 | - | - | nein |
| Klebung Bottom | bGlue | 36 | - | - | nein |
| DK Bohrungen | Drills | 44 | - | - | Ja |
| NDK Bohrungen | Holes | 45 | - | - | Ja |
| Innenfräsung | Milling | 46 | - | - | Ja |
| Abmessungen | Measures | 47 | - | - | Nein |

| eigene Lagen | Lagenname EAGLE | Lagennr. EAGLE | Abw. Lagenname | Abw. Lagennr. | Verw. |
| --- | --- | --- | --- | --- | --- |
| Bestückungsdruck Top |  |  | tprint | 112 | Ja |
| Bestückungsdruck Bottom |  |  | bnames | 26 | Ja |

Alle Lagen entsprechen der Eagle-Standard-Einstellung konvertieren.

Zuordnung Gerberdaten

| Gerber.XXX | Beschreibung | Verw. |
| --- | --- | --- |
| *.cmp | Top + Pads + Vias, Kupfer Leiterplattenoberseite | Ja |
| *.ly2 | Route2 + Pads + Vias, Kupfer Innenlage 1 | Ja |
| *.l15 | Route3 + Pads + Vias, Kupfer Innenlage 2 | Ja |
| *.sol | Bottom + Pads + Vias, Kupfer Leiterplattenunterseite | Ja |
| *.crc | tCream, Lotschablone Oberseite | Ja |
| *.crs | bCream, Lotschablone Unterseite | Ja |
| *.plc | Bestückungsdruck Top, Dimension | Ja |
| *.pls | Bestückungsdruck unten, Dimension5 | Ja |
| *.stc | tStop, Lötstopp Oberseite | Ja |
| *.sts | bStop, Lötstopp Unterseite | Ja |
| *.mil | Fräsung | Ja |
| *dri, drd | drill | Ja |